NATIONAL UNIVERSITY OF MODERN LANGUAGES

Computer Networks lab Report - 12

NAME: Abdul Rafay

ROLL NO: FL23791

PROGRAM:  BSSE (5th Semester)

COURSE: Computer Networks

SUBMITTED TO: Mam Anam Taskeen

DATE: 15, Sep 2025				 			 DAY: Monday

# Setting up Dynamic Routing Protocol (RIP Version 1 & Version 2)

Objectives

The objectives of this lab are:

- To configure router hostnames.

- To set passwords for router security.

- To configure three Cisco routers.

- To implement dynamic routing using RIP protocol.

- To understand the difference between RIP v1 and RIP v2.

Theoretical Description

By default, Cisco routers use generic hostnames such as Router, which makes network management difficult. Assigning meaningful hostnames helps in identifying devices easily during configuration and troubleshooting.

Router security is implemented by setting:

- Enable password

- Enable secret

- Console line password

- Auxiliary line password

The enable secret password has higher priority than the enable password and is encrypted.

In large networks, static routing becomes complex and difficult to manage. Therefore, dynamic routing protocols are used to automate route learning.
Routing Information Protocol (RIP) is a distance-vector routing protocol that uses hop count as its routing metric. The maximum hop count supported by RIP is 15, making it suitable for small networks.

RIP Version 2 improves upon RIP Version 1 by:

- Supporting classless routing (VLSM)

- Providing authentication

- Using multicast updates instead of broadcast

Lab Tasks and Configuration

This lab consists of multiple configuration tasks performed on three routers.

Task 1: Configure Hostname

Each router is assigned a unique hostname to improve identification.

Commands

Router(config)# hostname R1

The router name changes immediately after executing this command.

Task 2: Configure Enable Password

An enable password is set to restrict access to privileged EXEC mode.

Commands

R1(config)# enable password cisco

Task 3: Configure Enable Secret

Enable secret provides a more secure encrypted password.

Commands

R1(config)# enable secret cisco

Note: Enable secret overrides enable password.

Task 4: Configure Console Line Password

Console access is secured using a password.

Commands

R1(config)# line console 0

R1(config-line)# password lab

R1(config-line)# login

This ensures authentication when accessing the router through the console.

Task 5: Configure Auxiliary Line Password

Auxiliary line is secured for remote access.

Commands

R1(config)# line auxiliary 0

R1(config-line)# password ciscolab

R1(config-line)# login

Note: Auxiliary line password cannot be fully simulated in Packet Tracer.

Task 6: Configure IP Addresses on Router Interfaces

Each router interface is assigned an IP address according to the network topology.

Sample Commands

R1(config)# interface FastEthernet0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

The same steps are repeated for all routers and interfaces.

Task 7: Enable RIP Routing Protocol

RIP routing protocol is enabled on all routers.

Commands

R1(config)# router rip

Task 8: Configure RIP Version 2

RIP Version 2 is enabled to support classless routing.

Commands

R1(config-router)# version 2

Task 9: Advertise Networks

All directly connected networks are advertised using RIP.

Commands

R1(config-router)# network 192.168.1.0

R1(config-router)# network 192.168.2.0

Task 10: Disable Auto Summarization

Auto-summarization is disabled to allow proper subnet advertisement.

Commands

R1(config-router)# no auto-summary

Task 11: Verify RIP Routes

The routing table is checked to verify RIP-learned routes.

Commands

R1# show ip route

Routes learned via RIP are indicated with R.

Example:

R 192.168.2.0/24 [120/1] via 192.168.1.2

Result

- All routers were successfully configured with hostnames and passwords.

- RIP Version 2 was enabled on all routers.

- Routers dynamically learned routes using RIP protocol.

- Auto-summarization was disabled successfully.

Conclusion

In this lab, dynamic routing was implemented using RIP protocol. Router security was enhanced using passwords, and RIP Version 2 was configured to support modern networking features. The lab demonstrated how RIP simplifies routing in small-scale networks.