NATIONAL UNIVERSITY OF MODERN LANGUAGES

Computer Networks lab Report - 8

NAME: Abdul Rafay

ROLL NO: FL23791

PROGRAM:  BSSE (5th Semester)

COURSE: Computer Networks

SUBMITTED TO: Mam Anam Taskeen

DATE: 15, Sep 2025				 			 DAY: Monday

# Switch Redundant Connections and Loops (Spanning Tree Protocol – STP)

Objectives

The objectives of this lab are:

- To understand the concept of Spanning Tree Protocol (STP).

- To study how STP prevents loops in redundant switch networks.

- To configure a Root Bridge manually.

- To enable Rapid Spanning Tree Protocol (RSTP).

- To analyze root, forwarding, and blocked ports in a switched network.

Introduction

In modern network topologies, switches are often connected using redundant links to improve reliability and avoid a single point of failure. While redundancy increases availability, it also introduces network loops, which can cause broadcast storms and network failure.

To overcome this problem, Spanning Tree Protocol (STP) is used. STP logically disables redundant links and creates a loop-free tree topology, while still keeping backup paths available.

At the center of the STP network is the Root Bridge, which is the most important switch. All other switches calculate the best path to reach the Root Bridge and block unnecessary links.

Root Bridge Selection

The Root Bridge is selected based on the Bridge ID, which consists of:

- Priority number

- MAC address

By default:

- All switches have the same priority value (32768)

- The switch with the lowest MAC address becomes the Root Bridge

Since MAC addresses cannot be changed, we influence root selection by changing the priority value.
Lower priority = higher chance of becoming Root Bridge.

Priority values:

- Range: 0 to 61440

- Must be a multiple of 4096

Rapid Spanning Tree Protocol (RSTP)

RSTP is an improved version of STP that provides:

- Faster convergence

- No dependency on long timers

- Rapid recovery from topology changes

RSTP supports a maximum network diameter of 40 switches and is commonly used in modern networks.

Lab Tasks

Task 1: Configure Switch 1 as Root Bridge

To make Switch 1 the Root Bridge for VLAN 1, the following command is used:

Switch(config)# spanning-tree vlan 1 priority 0

OR

Switch(config)# spanning-tree vlan 1 root primary

This sets the lowest priority, ensuring Switch 1 becomes the Root Bridge.

Verification of Root Bridge

To verify the Root Bridge and port roles, the following command is used:

Switch# show spanning-tree

This command displays:

- Root Bridge information

- Port states (Root, Designated, Forwarding, Blocked)

- Path cost

Subtask: Switch Port Configuration

All switch ports are configured as trunk ports.

Port Configuration Requirements

- Ports: FastEthernet 0/1 – 0/5

- Speed: 100 Mbps

- Duplex: Full

- VLAN: 1

- Network: 192.168.1.0

- VLAN state: No shutdown

Task 2: Enable Rapid Spanning Tree Protocol (RSTP)

RSTP is enabled on all switches using the following command:

Switch(config)# spanning-tree mode rapid-pvst

This enables faster convergence and improved performance.

Checking Port States

To identify:

- Root Ports

- Forwarding Ports

- Blocked Ports

The following command is used:

Switch# show spanning-tree

Blocked ports confirm that STP is successfully preventing loops.

Result

- Switch 1 was successfully selected as the Root Bridge.

- Redundant links were automatically blocked to prevent loops.

- Rapid Spanning Tree Protocol was enabled on all switches.

- Network remained loop-free while maintaining redundancy.